Investigating Thermodynamic and Viscosity properties of Sample Liquids via Ultrasonic and Density Data.

Dhirendra Kumar Sharma1* and Suneel Kumar2

1,2 Department of Chemistry, Institute of Basic Science, Bundelkhand University, Jhansi (U.P), India.

*Corresponding author (e-mail:dhirendra.dr@rediffmail.com)

Mobile no. 9935328601

ABSTRACT:

Experimental values of ultrasonic velocity (u), density (ρ) and viscosity (𝜂) for the binary mixture 1,4 –Dioxane (Cyclic ether)  and Benzene, Ethyl benzene, Toluene were measured over entire composition range. The measurements were done at constant temperature 298.15 K and at ultrasonic frequency 3Mh.Z. From the experimental data excess sound velocity (uE) and excess viscosity (𝜂𝐸), were calculated. These results have been fitted to the Redlich-Kister polynomial equation. Excess viscosity (𝜂𝐸) and sound velocity (uE) were found to be negative for temperature 298.15K. Excess sound velocity (uE) and excess viscosity (𝜂𝐸), were plotted against the mole fraction of 1,4 –Dioxane over the whole composition range at 298.15 K. They have been analysed to discuss the nature and strength of intermolecular interactions in these mixtures.

Keywords: Ultrasonic velocity, Density, Viscosity, Binary liquid mixtures, cyclic ether, Redlich-Kister polynomial equation, Intermolecular interactions.

Graphical Abstract:

1. Introduction: Cyclic ether (1,4-Dioxane) and aromatic hydrocarbons are important organic solvents that can be used in medical, petrochemical and pharmaceutical industries to a great extent [1-5]. The ultrasonic measurements have been widely used to study the molecular structure moreover, they provide useful information on molecular interactions required for optimizing thermodynamic model development. Ultrasonic velocity is one such property which is very important for the study of molecular interactions in solution, its gives information about the extent of interaction and some other parameters such as adiabatic compressibility and free length, velocity determination of ultrasound waves enables us to study the properties of the medium, in which it travels. Owing to the importance of ultrasound velocity in the investigation of molecular interactions in solution. Its measurements have been carried out in the different binary liquid mixtures studied in the present work. Recently, ultrasonic method has become a powerful tool to provide information regarding the physical and chemical properties of liquid system. The ultrasonic study in organic liquid mixture is interesting to discuss non-linear behaviour with respect to concentration and frequency. The ultrasonic study of pure liquids and liquid mixtures is essential in understanding the nature of molecular interactions. The thermodynamic and acoustical properties of binary liquid systems have been studied for several reasons. The most important of these is which gives us information about the molecular interactions. The viscosity of binary liquid systems is one of the properties widely used in various chemical-engineering fields regarding the fluid flow, mass and heat transfer calculations. This property is also used for characterization of pure components. The present investigations are related to ultrasonic study on the binary liquid mixture of 1,4 –Dioxane (Cyclic ether) and Benzene, Ethyl benzene, Toluene. In this work, density, viscosity and ultrasonic velocity of 1,4 –Dioxane (Cyclic ether)  and Benzene, Ethyl benzene, Toluene binary liquid mixture have been measured at 298.15K and 3 Mh.Z. The excess thermodynamic properties provide useful information regarding intermolecular interactions between the component molecules of binary liquid mixtures [6-10]. These properties are of significance in theoretical and applied areas of research and these results are frequently used in designing of many chemical and industrial processes [11-13]. In almost all the models proposed in the literature for the liquid state, the cohesive forces are of major importance. The variations in the volumetric, acoustic properties of binary mixtures of cyclic ether with aromatic hydrocarbons with the molecular size, shape, chain length and degree of molecular association. In an attempt to explore the nature of interactions occurring between the mixing components, the densities, ρ, speeds of sound, u, viscosities, η, of binary mixtures of 1,4-dioxane with benzene, ethyl benzene and toluene have been measured over the entire range of composition at 298.15 K. From the experimental values of the density, speed of sound and viscosity, the values of the excess sound velocity, uE, and excess viscosity, ηE have been calculated. The calculated excess and deviation functions have been analyzed in terms of molecular interactions and structural effects. Binary mixture of 1,4 –Dioxane (Cyclic ether) with aromatic hydrocarbons is of great interest due to their extremely non-ideal behaviour and practical importance to the chemical industry. A lot of work has been reported in the literature. The studies of viscosity, density and ultrasonic velocity are being increasingly used as tools for investigation of the properties of pure components and the nature of intermolecular interaction between the liquid mixture constituents.

2. Experimental:

2.1 Chemicals / Materials.

The chemicals used in the present work were high purity laboratory reagent grade samples of 1,4-dioxane, Benzene Ethyl benzene and  Toluene, were supplied by CDH Ltd. New Delhi, India with purity 99.5%. All chemicals was stored over sodium hydroxide pellets for several days and fractionally distilled twice. All the chemicals were stored in dark bottles over freshly activated molecular sieve to minimize adsorption of moisture. The purity of the solvent was ascertained by comparing the measured density, dynamic viscosities and sound velocity of the pure component at 298.15K with the available literature [14-18] as shown in Table 2. The reported experimental values of density (ρ), viscosity (η) and sound velocity (u) conform closely to their corresponding literature values, with an average of the absolute value of deviation 3.6 × 10-3 kg m-3 and 3.3×10-3 m. Pa. s.

Table 1. CAS Registry Number, Mass Fraction Purity of the chemicals.

| Component | Formula | CAS Reg. No. | Supplier | Mass Fraction Purity (%) | Water Content | Method Purity analysis method |
| --- | --- | --- | --- | --- | --- | --- |
| 1,4-Dioxane | C4H8O2 | # 123-91-1 | CDH, (P) Ltd. New Delhi, India | 99.9% | 0.1% | Double distillation |
| Benzene | C6H6 | 108-67-8 | CDH, (P) Ltd. New Delhi, India | 98.0% | 0.01% | Double distillation |
| Ethyl benzene | C8H10 | 100-41-4 | CDH, (P) Ltd. New Delhi, India | 98.0% | 0.1% | Double distillation |
| Toluene | C7H8 | 108-88-3 | CDH, (P) Ltd. New Delhi, India | 99.0% | 0.1% | Double distillation |

Table-2: Comparison of Experimental and Literature density (ρ), speed of sound (u) and refractive index (n) of pure Components with Available Literature Values at T = 298.15K.

| Name of Liquid | Density (ρ) g.cm-3 |  | Viscosity (η) m.Pa.s |  | Sound Velocity (u) m.s-1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Experimental | Literature | Experimental | Literature | Experimental | Literature |
| 1, 4 Dioxane | 1.0212 | 1.024614 | 1.1285 | 1.194417 | 1346 | 134415 |
| Benzene | 0.8672 | 0.873615 | 0.5851 | 0.596115 | 1292 | 129915 |
| Ethyl Benzene | 0.8674 | 0.862016 | 0.6299 | 0.62816 | 1324 | 131216 |
| Toluene | 0.8576 | 0.862315 | 0.6026 | 0.552518 | 1306 | 130715 |

2.2 Measurements:

2.2.1 Apparatus and Procedure

Air tight stopper bottles were used for the preparation of the mixtures and were placed in the dark place. The losses in the mixtures were kept to minimum, as evidenced by repeated measurements of physical properties over an interval of 2-3 days during in which before use time no change in physical properties was observed. The mixtures were well mixed by shaking before use. Binary mixtures were prepared by mass, using an electronic analytical balance (Model K-15 Deluxe, K Roy Instruments Pvt. Ltd.) with an accuracy of ± 0.00001×10-3 kg as described elsewhere. The possible error in the mole fraction was estimated to be less than 1×10 -4. Five samples were prepared for one system, and their density and sound velocity were measured on the same day.

2.2.2 Density:

Densities of pure liquids and their binary mixtures were determined by using a R. D. Bottle with a 25 cm3 is used to measure the densities (ρ) of pure liquids and binary mixtures. The R. D. Bottle is calibrated by using conductivity water (having specific conductance less than 1×106 ohm-1) with 0.9970 and 0.9940 gcm-3 as its densities at T = 298.15 K, respectively. The R. D. Bottle filled with air bubbles free liquids is kept in a thermostate water bath (MSI Goyal Scientific, Meerut, India) controlled with a thermal equilibrium. The precision of the density measurements was estimated to be ±0.0002 g cm-3.

2.2.3 Sound velocity: The ultrasonic velocity were measured using a multi-frequency ultrasonic interferometer (Model F80D, Mittal Enterprise, New Delhi, India) working at 3 M.Hz. The meter was calibrated with water and benzene. Measurement of sound velocity through medium was based on the accurate determination of the wavelength of ultrasonic waves of known frequency produced by quartz crystal in the measuring cell. The interferometer cell was filled with the test liquid, and water was circulated around the measuring cell from a water bath. The uncertainty was estimated to be 0.1 ms-1. The measured values of ultrasonic velocities of pure 1,4-dioxane, benzene,  ethyl benzene, and toluene at 298.15K were 1346, 1292, 1324, and 1306 m.s-1 respectively, which compare well with the corresponding literature values.

2.2.4 Viscosity:

The viscosity of pure liquids and their binary mixture were measured using suspended Ostwald viscometer having a capacity of about 15 ml and the capillary having a length of about 90 mm and 0.5 mm internal diameter has been used to measure the flow time of pure liquids and liquid mixtures and it was calibrated with triply distilled water, methanol and benzene at 298.15 K. The details of the methods and techniques have been described by researchers [19-20]. The efflux time was measured with an electronic stop watch (Racer) with a time resolution (±0.015), and an average of at least four flow time readings was taken. Glass stopper was placed at the opening of the viscometer to prevent the loss due to evaporation during measurements. The two bulbs reservoir, one at the top and other at the bottom of the viscometer linked to each other by U type facilitate the free full of liquid at atmospheric pressure. Viscosity values (η) of pure liquids and their binary mixtures are calculated using the solution.

ηρ = at - bt					(1)

Where t is the efflux time and a and b are viscometric constants.

The measured viscosities have reproducibility within ±0.002 m.Pa.s. The measured values of viscosities of pure 1,4-dioxane, benzene,  ethyl benzene, and toluene at 298.15 K were 1.1285, 0.5851, 0.6299, 0.6026 mPas . which compare well with the corresponding literature values.

3.0 THEORETICAL:

The excess sound velocity (uE) is evaluated from the experimental values of ultrasound velocities for component liquid and their binary mixtures by

uE  =  u1,2  - u1X1 + u2 X2                                                 (2)

Where u1,2 is ultrasound velocity in the mixture and u1, u2, X1, X2 are the sound velocities and mole fractions respectively of the component liquid 1 and 2.

The ultrasonic velocity (u), density (ρ) and viscosity (η) in pure liquids and liquid mixtures of various concentrations have been measured at 298.15 K.

3.1 Viscosity Deviations:

The viscosity deviations (Δ𝜂) with mole fraction were calculated by the following:

2

Δ 𝜂 = 𝜂12 −  ∑  𝑖𝜂𝑖,                                                        (3)

𝑖=1

Where, 𝑥𝑖, 𝜂𝑖, and 𝜂12 refer, respectively, to the mole fraction and viscosities of 𝑖th pure components and of the binary mixtures.

The excess value of AE of these thermodynamic parameters have been obtained by subtracting the ideal value from the experimental value

AE = Aexp. – (X1 A1+ X2 A2)

Where A represents the parameter such as intermolecular free length, molar volume, available volume, free volume and isentropic compressibility and X1 and  X2 are the mole fractions of components whose parameters.

4.0 Results and Discussion

The experimental values of densities, sound velocity and viscosities of the hydrocarbons are compared with the literature values and are presented in Table 2. It was found that the experimental values are in proximity with the literature values. Insufficient data on densities, viscosities and sound velocity of pure1,4-dioxane, Benzene, Ethyl benzene, Toluene, is available. The densities, 𝜌, viscosities, 𝜂, and sound velocity, u, of binary mixtures were measured at 298.15 ± 0.01 K as a function of the composition of the corresponding binary mixtures. The results of the study are presented in Tables 3. A perusal of table 3 shows that the sound velocity increase with mole fraction of 1,4-dioxane increases for all the binary mixtures. Ultrasonic wave are high frequency mechanical waves. Their velocities in a medium depend inversely on density and the compressibility of the medium.

Table-3 shows that the density of the binary mixtures increase with increasing mole fraction of the 1,4-dioxane. This trend may suggest that the increasing concentration of the 1,4-dioxane leads to increase the number of particles in a given region [21]. The increasing trend of viscosity with increasing mole fraction of 1,4-dioxane is due to that the intermolecular interaction between the 1,4-dioxane aromatic hydrocarbons. The increasing in density and viscosity with increase in concentration of 1,4-dioxane indicating loosening of intermolecular forces due to thermal agitation of molecules in the mixture. In this present study, ultrasonic velocity increasing with increasing concentration of 1,4-dioxane (Table-3). Moreover acoustic impedance shows the same trend of ultrasonic velocity. It may be due to the structural changes occurring in the mixtures resulting in weakening of intermolecular forces.

1,4-dioxane +  aromatic hydrocarbons (Binary Mixture)

1,4-dioxane            Aromatic Hydrocarbon

Results

Speed of sound measurement

Scheme 1. Interactions between1,3-dioxolane with 1-alkanols at 298.15K

Scheme 1. Interactions between1,4-dioxane with aromatic hydrocarbons at 298.15K

For the binary liquid mixture 1,4-dioxane (1) + Benzene (2), 1,4-dioxane (1) + Ethyl Benzene (2) and 1,4-dioxane (1) + Toluene (2), the obtained excess sound velocity (uE) values are negative over the whole composition range at 298.15 K as depicted in figure-1. Various type of interaction which are possible and which can operate in the binary liquid mixtures containing 1,4- dioxane and aromatic hydrocarbon, that can produce negative deviation in excess sound velocity and excess viscosity.

The excess sound velocity (uE) for the binary systems of 1,4-dioxane (1) + Benzene (2), 1,4-dioxane (1) + Ethyl Benzene (2) and 1,4-dioxane (1) + Toluene (2)  are graphically presented in Fig. 1, together with the Redlich-Kister correlation. It is evident that all three mixtures exhibit significant negative uE values, across the whole composition range and at working temperature 298.15K.

Figure-1: Plots of excess sound velocity (uE) versus mole fraction of 1,4-Dioxane (x1) at 298.15K for binary mixtures of 1,4-Dioxane with Benzene (◊), Ethyl benzene (□) and Toluene (Δ).

The nature of molecular interactions between the components of the liquid mixtures, it is of interest to discuss the same in terms of excess parameters rather than actual values. Non-ideal liquid mixtures show considerable deviation from linearity in their concentrations and this can be interpreted as the presence of strong or weak interactions. The extent of deviation depends upon the nature of the constituents and composition of the mixtures. The thermodynamic excess properties are found to be more sensitive towards intermolecular interaction among the component molecules of liquid mixtures. The sign and extent of deviation of excess parameters depend upon on the strength of interaction between unlike molecules (22). So various excess acoustic and thermodynamic parameters have been evaluated and corresponding graphs are also given.

The sign and magnitude of excess ultrasonic velocity (uE) play an important role in describing molecular rearrangement as a result of the molecular interaction between the component molecules in the mixtures. The excess ultrasonic velocity (uE) curves at 298.15 K varying with mole fraction of 1,4-dioxane are represented in Figure-1 for the three binary systems. The excess ultrasonic velocity values exhibiting negative in all three binary systems. Generally, the value of the excess function (uE) depend upon several physical and chemical contributions (23,24). The physical contribution depends mainly on two factors, namely:

1. The dispersion forces or weak dipole-dipole interaction that leads to positive values.

2. The geometrical effect allowing the fitting of molecules of two different sizes in to each other’s structure resulting in negative values

# Table 3. Values of density (ρ), sound velocity (u) and viscosity (η)   for Various 1,4-Dioxane Mole Fractions x1 of the Binary Mixture (1,4-Dioxane (1) + Aromatic hydrocarbon (2)) at Temperatures T = 298.15 K.

| Mole fraction of 1,4-dioxane (𝑥1) | Density ( ρ) g.cm-3 | Sound velocity (u) m.s-1 | Viscosity ( η) m.Pa.s |
| --- | --- | --- | --- |
| 1,4-dioxane + Benzene |  |  |  |
| 0.0000 | 0.8672 | 1292 | 0.5851 |
| 0.1024 | 0.8684 | 1296 | 0.6301 |
| 0.2117 | 0.8752 | 1302 | 0.6618 |
| 0.3214 | 0.8894 | 1307 | 0.7012 |
| 0.4617 | 0.9026 | 1311 | 0.7526 |
| 0.5001 | 0.9326 | 1317 | 0.7915 |
| 0.6616 | 0.9424 | 1324 | 0.8125 |
| 0.7088 | 0.9621 | 1328 | 0.8498 |
| 0.8079 | 0.9860 | 1334 | 0.9750 |
| 0.9026 | 1.0084 | 1340 | 1.0441 |
| 1.0000 | 1.0212 | 1346 | 1.1285 |
| 1,4-dioxane + Ethyl Benzene |  |  |  |
| 0.0000 | 0.8674 | 1324 | 0.6299 |
| 0.1024 | 0.8692 | 1326 | 0.6471 |
| 0.2117 | 0.8780 | 1328 | 0.6741 |
| 0.3214 | 0.9080 | 1330 | 0.7288 |
| 0.4617 | 0.9208 | 1332 | 0.7927 |
| 0.5001 | 0.9404 | 1334 | 0.8314 |
| 0.6616 | 0.9548 | 1336 | 0.8886 |
| 0.7088 | 0.9764 | 1338 | 0.9246 |
| 0.8079 | 0.9924 | 1340 | 0.9597 |
| 0.9026 | 0.9992 | 1342 | 0.9880 |
| 1.0000 | 1.0212 | 1346 | 1.1285 |
| 1,4-dioxane + Toluene |  |  |  |
| 0.0000 | 0.8576 | 1306 | 0.6026 |
| 0.1024 | 0.8708 | 1312 | 0.6564 |
| 0.2117 | 0.8816 | 1319 | 0.7101 |
| 0.3214 | 0.8976 | 1325 | 0.7365 |
| 0.4617 | 0.9026 | 1331 | 0.7789 |
| 0.5001 | 0.9336 | 1337 | 0.8181 |
| 0.6616 | 0.9676 | 1340 | 0.8667 |
| 0.7088 | 0.9732 | 1342 | 0.9150 |
| 0.8079 | 0.9837 | 1344 | 0.9590 |
| 0.9026 | 0.9932 | 1345 | 0.9980 |
| 1.0000 | 1.0212 | 1346 | 1.1285 |

The chemical contributions include breaking up of the associates present in pure liquids, resulting in negative  uE. The ultrasonic velocity in a mixture is mainly influenced by its molecular property. The results for the excess sound velocity (uE) plotted in figure-1 are negative for all the binary system studied. The observed negative trends in excess sound velocity indicate that the effect due to the breaking up of self-associated structure of the components of the mixtures is dominant over the effect of dipole-dipole interaction between unlike molecule. The negative values of excess sound velocity (uE) increase with the increase in mole fraction which indicates the increase in strength of interaction with all three binary mixtures. The negative excess sound velocity (uE) clearly suggests that there exist strong molecular interaction between the molecules of all the three binary liquid mixtures.

The measurement of viscosity in binary liquid mixture gives some reliable information in the study of intermolecular interaction. The molecules of one or more components forming the temarise are either polar, associating or accordingly show non-ideal behavior’s in mixtures. Negative values of ηE  in most of the cases are the consequence of lower viscosity contributions of similar non-specific interaction and hydrogen bonding effect of molecular species in real mixtures rather than those in the corresponding ideal  mixtures. In the present study, it is observed that, for the three binary systems the ηE  values gradually decrease up to the mole fraction around 0.5 and then begins to increase Figure 2 more over it is observed that the ηE values decrease as the concentration of x1 increase. The negative values imply the presence of dispersion forces between the mixing components in the mixtures.

The excess transport properties of the mixtures are influenced by three main types of contributions, namely,

- due to non specific Van der Waals type forces,

- due to hydrogen bonding, dipole-dipole, and donor-acceptor interaction between unlike molecules, and

- due to the fitting of smaller molecules into the voids created by the bigger molecules.

The experimental values of viscosities as a function of mole fraction of cumene for three systems are shown in Fig 2. The three systems exhibit a negative deviation of excess viscosity over entire mole fraction range with a maximum corresponding to a mole fraction of about 0.5 at the temperature studied. These deviations indicate specific molecular interactions between different molecules. According to Fort and Moore, the excess viscosity gives the strength of the molecular interaction between the interacting molecules. The excess value of viscosity at the three binary mixtures 1,4-dioxane (1) + Benzene (2), 1,4-dioxane (1) + Ethyl Benzene (2) and 1,4-dioxane (1) + Toluene (2) at the 298.15 K are shown in figure - 2. The Figure -2 represents the variation of excess viscosity (ηE) is found to be negative for all three binary liquid mixtures over the entire composition range at the 298.15 K. Which suggest the presence of weak intermolecular interactions. For systems where dispersion, induction and dipolar forces are operating, the values of excess viscosity are found to be negative, whereas the existence of specific interaction leading to the formation of complexes in mixtures tends to make negative.

Figure-2: Plots of excess viscosity (ηE) versus mole fraction of 1,4-Dioxane (x1) at 298.15K for binary mixtures of 1,4-Dioxane with Benzene (◊), Ethyl benzene (□) and Toluene (Δ).

The viscosities of binary liquid mixture 1,4-dioxane (1) + Benzene (2), 1,4-dioxane (1) + Ethyl Benzene (2) and 1,4-dioxane (1) + Toluene (2),  at temperature T = 298.15 K decrease linearly with increase in mole fraction of 1,4-Dioxane. Excess viscosity (ηE) values are negative for the all three binary system over the whole mole fraction range (Figure-2).

A reduction in viscosity with increase in mole fraction of 1,4-dioxane suggests that the existing intermolecular interactions are weakening in magnitude. However, the increasing sound velocity and density with increasing mole fraction of 1,4-dioxane leads to a notion that the system is getting more and more compact, which is not true as the interactions due to 1,4-dioxane are dispersive in nature.

The excess viscosity values, which represent the deviation from rectilinear dependence of ηexp of binary mixture on mole fraction, have been used to explain the mixture component’s intermolecular interaction. The negative ηE values might indicate that the average degree of cross-association between aromatic hydrocarbon and 1,4-dioxane gradually decreased as the chain length of hydrocarbon increased. Thus, the larger negative deviation for the system containing longer chain hydrocarbon confirmed strong dispersion forces in this system. It can be seen from figure-2 that in the mixture, absolute value of (ηE) decrease at mole fraction in raised. An increment of temperature diminishes the self association of the pure component and also the hetro association between unlike molecule, because of the increase of the thermal energy. This lead to less negative values of Δη as temperature is raised as observed in the present binary mixture [25-26]. Many workers have reported similar behaviour where negative value of (ηE) indicates dispersive interaction. The negative values of excess viscosity (ηE) observed in 1,4-dioxane (1) + Benzene (2) mixture indicate the presence of strong inter molecular interaction amongst the mixing components. The values of excess viscosity (ηE) for all the three system studied are indicative of the predominance of dispersion forces.

Conclusion: In this paper the ultrasonic velocity (u), density (ρ) and viscosity (η) have been measure over the whole composition range at temperature T = 298.15 K for the binary mixture1,4-Dioxane, Benzene, Ethyl benzene, Toluene. Excess viscosity, excess sound velocity for binary mixtures have been calculated and fitted to a Redlich–Kister equation. It is obvious that, there exist a molecular interaction between the components of the mixture. In specific weak molecular interaction like dipole-dipole, dipole-induced dipole and dispersive forces are found to exist between the components of the individual mixtures.

Acknowledgement: The authors are very much thankful to the Hon’ble Vice Chancellor, Prof. Mukesh Pandey, Bundelkhand University, Jhansi (U.P.) India. For proving the facilities for Research work.

Declarations conflict of interest

The authors have no competing interests to declare that are relevant to the content of this article.

Funding

The author(s) reported there is no funding associated with the work featured in this article.

Data availability statement

All data generated or analyzed during this study are included in this published article.

REFERENCES

(1) M. V. Rathnam, D. R.  Ambavadekar, M. Nandini, “Densities, viscosities, and sound speed of binary mixtures of   hexylacetates with tetra hydro furan, 1, 4-dioxane, anisole, and butyl vinyl ether”. J. Chem. Eng. Data, Vol. 58, p.p.3370−3377, 2013.

(2) K. Rajgopal, S. Chenthilnath, “Study on excess thermodynamic parameters and theoretical estimation of ultrasonic velocity of 2-methyl 2-propanol with nitriles at different temperatures”. J. Chem. Eng. Data, Vol. 18, p.p.806−816, (2010).

(3) K. N. Anil, “Ultrasonic and viscometric study of molecular interactions in binary mixtures of aniline with 1-propanol, 2-methyl propanol at different temperatures.” Fluid Phase Equilib. Vol. 259 p.p. 218−227, (2007).

(4) P. Shahla, S. Divya, S. Shashi, K. P Singh, G.  Manisha, J. P. Shukla, “Ultrasonic velocity, density, viscosity and their excess parameters of the binary liquid mixtures of tetra hydro Furan with methanol and o- cresol at varying temperatures”. Appl. Acoust. Vol.70, p.p. 507−513 (2009).

(5) K. Rajgopal, S. Chenthilnath, “Excess parameters studies on binary liquid mixtures of 2-methyl 2-propanol with aliphatic nitriles at different temperatures.” J. Mol. Liq. Vol. 160 p.p. 72−80, (2011).

(6) G.P.  Dubey, S. Rani,P. Kaur, “Study of molecular interactions in binary liquid mixtures containing tri n-butylamine with 2-pentanone, 3-pentanone, and 4-methyl 2-pentanone: A thermo physical approach” J. Mol. Liq., Vol. 222.p.p. 415, (2016).

(7) N. Chaudhary, A.K. Nain, “Densities, ultrasonic speeds, viscosities, refractive indices and excess properties of 1-butyl-3-methylimidazolium tetra fluoro borate + N-methyl acetamide binary mixtures at different temperatures”, J. Chem. Eng. Data. Vol. 65, p.p. 1447, (2020)  .

(8) N. Chaudhary, A.K. Nain, “Volumetric, ultrasonic, viscometric and refractive index studies of molecular interactions in binary mixtures of 1-butyl-3-methylimidazolium tetrafluoroborate with methyl acrylate at temperatures from 293.15 to 318.15 K”, J. Mol. Liq., Vol. 297 , p.p.111890, (2020).

(9) C.M. Romero, C. Guzman, I.  Gascon, P. Cea,M.C.  Lopez, “Speeds of sound and isentropic compressibility for binary mixtures of a cyclic diether with a cyclic compound at three temperatures”, Int. J. Thermo phys., Vol. 27, p.p. 760, (2006).

(10) A. K. Nain , P. Droliya, J.  Gupta, Indian J. Chem. A, Vol. 57, p.p. 761, (2018) .

(11) S. Kumar, W. Lin, Y. Lee, J.S. Yadav, D. Sharma, V.K. Sharma, I. Moon, “Thermodynamic and acoustic properties for binary and ternary mixtures of cyclic ethers with industrially important solvents at 308.15 K”, J. Mol. Liq.,Vol. 155, p.p. 8, (2010)  .

(12) S. Mrad, C. Lafuente, B. Giner, M. Hichria, “Thermo physical study of the binary mixtures of N, N dimethylacetamide with 2-propanol and 2-butanol”, Thermo chim. Acta, Vol. 655, p.p.  169, (2017).

(13)I.  Bahadur, N. Deenadayalu, D. Ramjugernath , “Effects of temperature and concentration on interactions in methanol + ethyl acetate and ethanol + methyl acetate or ethyl acetate systems: Insights from apparent molar volume and apparent molar isentropic compressibility study”, Thermo chim. Acta, Vol. 577, p.p. 87, (2014).

(14) K. Taoufik, et al, “Denisty, speed of sound and refractive index measurements for the binary mixtures at T = (295.15,298.15,301.15,304.15,307.15,310.15 and 313.15) K”., Journal of Chemical and Engineering data. (2015).

(15) J.D. Pandey, et al, “nternal pressure, ultrasonic velocity and viscosity of multi-component liquid systems,” Pramana Journal of Physics, Vol. 40(2), p.p. 81-87, (1993).

(16) V.K. Rattan, S. Singh,P.S. Sethi, “Viscosities, Densities and Ultrasonic velocities of binary mixtures of Ethyl benzene with Ethanol, 1-Propanol and 1-Butanol at (298.15 and 308.15)K”. Journal of Chemical and Engineering data. Vol. 49,p.p.  1074-1077, (2004).

(17) P.K. Banipal, et al, “Physico-Chemical studies on binary mixtures of 1,4-dioxane and Akan-1-ols at 298.15K”, Proc. Acad. Sci., India, Sect. A Phys. Sci. Vol. 88(4), p.p.  479-490, (2018)  .

(18) K. Sarkar, Bipul et al, “Excess molar volume, excess viscosities and ultrasonic speed of sound of binary mixtures of 1,2-Dimethoxyethane with some aromatic liquids at 298.15K,” J. Solution chem., Vol.41, p.p. 53-74, (2012) .

(19) M.N. Roy, A. Jha,  R. Dey, “Study of Ion−Solvent Interactions of Some Alkali Metal Chlorides in Tetrahydrofuran + Water Mixture at Different Temperatures”,  J. Chem. Eng. Data, Vol. 46(5), p.p 1327–1329, (2001) .

(20) M.N. Roy, A. Jha, A. Choudhury, “Densities, Viscosities and Adiabatic Compressibilities of Some Mineral Salts in Water at Different Temperatures”, J. Chem. Eng. Data, Vol. 49(2), p.p. 291-296, (2004).

(21) P. Vasantharani, V. Pandiyan , A.N. Kannappan, Asian J. Appl. Sci. Vol.2 , p.p. 169, (2009)  .

(22) R.P. Singh, C.P.  Sinha J.C. Das, P. Ghosh,  Journal of Chem. Eng. Data, Vol.35, p.p. 93, (1990) . https://aip.scitation.org/doi/abs/10.1063/1.4926955

(23) T. Sumathi, S.  Govindarajan,  International Journal of Biology, Pharmacy and Allied Sciences (IJBPAS), Vol. 1, p.p. 1153, (2012).     https://www.hindawi.com/journals/ac/2014/343012/

(24) A. Ali, A.K. Nain, V.K. Sharma,S. Ahmed,  Indian Journal of Physics B, Vol.75, p.p.519, (2001). http://arxiv.iacs.res.in:8080/jspui/bitstream/10821/6364/1/Ultrasonic%20Studies%20in%20Binary%20Liquid%20Mixtures_A%20Ali.pdf

(25) P.S. Naidu, K.P. Ravindra, Indian J.  Pure & Appl. Phys.Vol.  40 , p.p. 264, (2002)  . http://nopr.niscpr.res.in/handle/123456789/25299

(26) G.V. Rama Rao, A. Viswanatha Sama, D. Ramachandran , C. Rambabu, Indian Journal of Pure & Applied Physics., Vol. 43, p.p. 602, (2005). http://nopr.niscpr.res.in/handle/123456789/8851